EM5230

Power Quality Analyzer

Power on main interface

Comprehensive parameters - 3-phase 4-wire test interface

Phase measurement - 3-phase 4-wire test interface

Unbalance - 3-phase 4-wire test interface

Product Function

- Comprehensive analysis and diagnosis of three-phase power quality.

- 6 types of current sensors for options to application on various sites.

- 4 current and 3 voltage channels are measured simultaneously.

- Testing accuracy is class 0.5, the test is stable and the anti-interference ability is strong.

- Touch colorful screen, simple human-machine operation, and switch with different operation language (Chinese & English).

- USB communication upload the test results to the computer for analysis.

- Equip with large capacity lithium battery.

- Waveform real-time display (3 voltage/4 currents channels).

- True effective values (TRMS) of voltage and current.

- Phase diagram display.

- Each phase harmonics measurement, harmonic up to 51 orders.

- The bar graph shows the harmonic ratio of each phase current and voltage.

- Total harmonic distortion (THD).

- Active/reactive/apparent power value and total value of each phase.

- Voltage & current power trend chart.

- Three-phase unbalance measurement (voltage & current).

- Record waveform and parameters.

- Screenshot and save.

- Touch screen operation.

- Voltage flicker, voltage surge and voltage drop capture.

- Starting current and surge current monitoring.

- USB interface communicate with the computer.

- Setting the wiring connection method and power network type of the instrument.

- Selecting different type clamp current sensor and different voltage test ratio.

- One key to phase volt-ampere testing, turns ratio testing, and DC testing.

- Power energy calculation.

- DC voltage,current and power.

- DC

Product Features

Technical Specification

| Power Supply | Rechargeable lithium battery 7.4V, 5200mAH, external charger |  |
| --- | --- | --- |
| Display Mode | 7 inch touch colorful screen |  |
| Channel | 3 voltage channels, 4 current channels |  |
| AC Voltage Range | AC 0.001V ~ 1000V |  |
| DC Voltage Range | DC 0.1V ~ 1000V |  |
| Clamp Current Sensor (Optional) | 008B Clamp current sensor: Ф8mm; 032B Clamp current sensor: Ф32 x 42mm 050B Clamp current sensor; Ф50mm | 300FA Flexible coil current sensor: Ф300mm 035AD DC current sensor: Ф30 x 35mm 068AD DC current sensor: Ф68mm |
| Current Range | 008B Clamp current sensor: AC 10mA ~ 10.0A 032B Clamp current sensor: AC 0.1A ~ 100A 050B Clamp current sensor; AC 1A~1000A | 300FA Flexible coil current sensor: AC 10A ~ 6000A 035AD DC current sensor: DC 0.1A ~ 1000A 068AD DC current sensor: DC 0.1A~1000A |
| Frequency | 40Hz ~ 70Hz |  |
| Harmonic | Yes, 2 ~ 51 orders |  |
| Total Harmonic Distortion | Yes, 2 ~ 51 orders, each phase |  |
| Memory | 8G, expandable to 32G |  |
| Three-Phase Unbalance | Yes |  |
| Communication Interface | USB |  |
| Working Current | Approx. 490mA |  |
| Battery Power Indication | Battery symbol display battery power, when the battery power too low, the instrument will automatic shutdown |  |
| Automatic Shut Down | When record function enable, the meter will not automatic shutdown |  |
|  | When record function disable, setting the automatic shutdown time of 15minutes, 30minutes, 60minutes. |  |
| Input Impedance | Test voltage input impedance: 1MΩ |  |
| Withstand Voltage | The instrument circuit and the housing can withstand 3700V/50Hz sine wave AC voltage for 1 minute. |  |
| Insulation | Between the instrument circuit and housing ≥ 10MΩ |  |
| Structure | Double insulation, with insulation anti-vibration sheath |  |
| Suitable Safety Standard | IEC-61010 CAT III 1000V / CAT IV 600V, IEC-61010-031, IEC-61326, Pollution Degree 2 |  |
| Working Temperature | - 10℃ ~ 40℃; Below 80%RH |  |
| Storage Temperature | - 10℃ ~ 60℃; Below 80%RH |  |
| Instrument Weight | Approx. 8kg (include all accessories) |  |
| Accessories | Host*1PC; Instrument tool bag*1PC; Pen probe test lines*4PCS (each 1PC of yellow, green, red, black); Alligator clip*4PCS; Power adaptor*1PC; 4G Memory card*1PC. Clamp current sensor*(optional) |  |

Instrument Accuracy

| Measurement Specification | Range | Display Resolution | Max Error |
| --- | --- | --- | --- |
| Frequency | 40Hz ~ 70Hz | 0.001Hz | ± (0.01)Hz |
| Voltage True RMS | 0.01V ~ 1000V | Min. resolution 0.001V | ± 0.5 % rdg ± 0.1 % fs |
| DC Voltage | 0.1V ~ 1000V | Min. resolution 0.1V | ± (0.5 % rdg ± 5dgt)) |
| Current True RMS | 0.01mA ~ 6000A | Min. resolution 0.001A | ± (0.5 % rdg ± 5dgt)) |
| DC Current | 0.1A ~ 1000A | Min. resolution 0.1A | ± (0.5 % rdg) |
| Active Power | 0.001W ~ 6000kW | Min. resolution 0.001W | ± (1 % rdg + 3dgt)；CosΦ ≥ 0.8 |
|  |  |  | ± (1.5 % rdg + 10dgt)；0.2 ≤ CosΦ < 0.8 |
| Reactive Power | 0.001VAR ~ 6000kVAR | Min. resolution 0.001VAR | ± (1 % + 3dgt); SinΦ ≥ 0.5 |
|  |  |  | ± (1.5 % + 10dgt); 0.2 ≤ SinΦ < 0.5 |
| Apparent Power | 0.001VA ~ 6000kVA | Min. resolution 0.001VA | ± (1 % + 3dgt %) |
| Power Factor | - 1.000 ~ 1.000 | 0.001 | ± (1.5 % + 3dgt); CosΦ ≥ 0.5 |
|  |  |  | ± (1.5 % + 10dgt); 0.2 ≤ CosΦ < 0.5 |
| Active Energy | 0.001kWh ~ 9999.999MWh | Min. resolution 0.001kWh | ±(1 % rdg + 3dgt)；CosΦ ≥ 0.8 |
|  |  |  | ± (1.5 % rdg + 10dgt)；0.2 ≤ CosΦ < 0.8 |
| Reactive Energy Inductive & Capacitive | 0.001kVARh ~ 9999.999MVARh | Min. Resolution 0.001kVARh | ± (1 % rdg + 3dgt)；SinΦ ≥ 0.5 |
|  |  |  | ± (1.5 % rdg + 10dgt)；0.2 ≤ SinΦ < 0.5 |
| Apparent Energy | 0.001kVAh ~ 9999.999MVAh | Min. Resolution 0.001kVAh | ± (1 % rdg + 3dgt) |
| Phase Angle | - 179° ~ 180° | 0.001° | ± (3°) |
| Voltage Harmonic Ratio 2 ~ 51 Orders (Vrms > 50V) | 0.0 % ~ 99.9 % | 0.001% | (2 ~ 20 orders) ± (1 % rdg + 5dgt) (21 ~ 30 orders) ± (1 % rdg + 10dgt) (31~51 orders)±(1%rdg+15dgt) |
| Voltage Harmonic Angle (Vrms > 50V) | -179° ~ 180° | 0.001° | ± (3°) harmonic 2 ~ 25 orders |
|  |  |  | ± (10°) harmonic 26 ~ 51 orders |
| Current Harmonic Ratio 2~51 Orders (Arms>I range/100) | 0.0 % ~ 99.9 % | 0.001% | (2 ~ 20 orders) ± (1 % rdg + 5dgt) (21 ~ 30 orders) ± (1 % rdg + 10dgt) (31 ~ 51 orders) ± (1 % rdg + 15dgt) |
| Current Harmonic Angle (Arms >Arms>I range/100) | -179° ~ 180° | 0.001° | ± (3°) harmonic 1 ~ 25 orders ± (10°) harmonic 26 ~ 50 orders |
| Total Harmonic Rate (DF or THD-F) ≤ 50 | 0.0 % ~ 99.9 % | 0.001% | ± (1 % rdg + 10dgt) |
| Distortion Factor (DF or THD-R) ≤ 50 | 0.0 % ~ 99.9 % | 0.001% | ± (1 % rdg + 10dgt) |
| Three-phase Unbalance | 0.0 % ~ 100 % | 0.01% | ± (1 % ) |

Current Sensor Characteristics (Optional)

| Current Sensor Model | Photo | Current True RMS | Max. Resolution | Current True RMS Max Error | Phase Angle φ Max Error |
| --- | --- | --- | --- | --- | --- |
| EM008B CT Size: Ф8mm |  | AC 10mA ~ 10.0A | 1mA | ± (1 % rdg + 0.05 % fs) | ± (2°) |
| EM032B CT Size: 35 x 40mm |  | AC 0.10A ~ 100A | 1mA | ± (1 % rdg + 0.05 % fs) | ± (2°) |
| EM050B CT Size: Ф50mm |  | AC 1.0A ~ 1000A | 1mA | ± (2 % rdg + 0.05 % fs) | ± (2°) |
| EM300FA CT Size: Ф300mm |  | AC 10.0A ~ 6000A | 1mA | ± (1 % rdg + 0.05 % fs) | ± (2°) |
| EM035AD CT Size：Ф30 × 35mm |  | DC 0.1A - 1000A | 0.1A | ± (3 % rdg + 0.5 % fs) |  |