EM5910

Intelligent Digital Multimeter

Product Function

- Measurement of AC voltage true effective value (TRMS), DC voltage, resistance, frequency, capacitance, temperature, continuity test, NCV, diode , etc.

- The portable 6000-count automatic range digital multimeter is compact in structure, easy to operate and convenient to carry. It is an ideal tool for electrical and electronic measurements.

- Equipped with large LCD display, which is convenient for the user to read the measured value in dark place.

- With automatic shutdown, data hold and a lighting lamp function.

Product Features

Technical Specification

| Power Supply | DC3V Alkaline dry battery (LR03 1.5V x 2PCS) |  |
| --- | --- | --- |
| LCD Size | 52mm x 67mm; Max display: 5999 |  |
| Voltage Measurement | Range: DC 6V, 60V, 600V, 1000V | ± (0.8 % + 3) |
|  | Range: AC 6V, 60V, 600V, 750V | ± (1.0 % + 3) |
| Resistance Measurement | Range: 600Ω, 6kΩ, 60kΩ, 600kΩ, 6MΩ, 60MΩ | ± (1.2 % + 3) |
| Capacitance Measurement | Range: 6nF, 60nF, 600nF, 6uF, 60uF, 600uF, 6mF | ± (3 % + 5) |
| Frequency Measurement | Range: 6Hz, 60Hz, 600Hz, 6kHz, 60kHz, 600kHz, 6Mhz | ± (0.1 % + 3) |
| Thermometry | Range: 0℃ ~ 1000℃ | ± (5.0 % + 4) |
| NCV Function | Yes |  |
| Backlight | Yes |  |
| Data Hold | Yes |  |
| Continuity Measurement | Yes |  |
| Diode Measurement | Yes |  |
| Automatic Shutdown | Automatic shutdown after starting up 14 minutes without any operation |  |
| Overflow Display | Over range overflow function: the highest digit display “OL” |  |
| Overload Protection | Yes |  |
| Input Impedance | 10MΩ |  |
| Battery Voltage | When battery power indicate “ ”symbol, remind to replace the battery |  |
| Working Temperature | 0℃ - 40℃ RH < 75 % |  |
| Store Temperature | - 10℃ - 60℃ RH < 80 % |  |
| Meter Weight | Approx. 146g (include accessories) |  |
| Meter Dimension | 142mm x 70mm x 20mm |  |
| Accessories | Meter*1PC; Probe*1SET Temperature probe*1PC; Tool bag*1PC; LR03 battery*2PCS |  |